# Hardware Design

## Controllers

ESP32-C6

## Power Source

Use USB A 5V as power source. funnel Power lead

## Component positions

### Controller

Controller will sit in socket of small compartment below the lowest shelf

### Door Switches

Door Switches are mounted under the topmost shelf (ceiling)

Each sits where the door meets the frame. The closer to the edge, away from the hinges, the quicker it reacts

#### LEDs

LEDs are clustered into blocks of 7 LEDs in two rows

Each cluster extends to behind the door frame outside of the window, on both sides.

Distances between LEDs must be uniform, laterally and sagitarially.

Each cluster is the width of a small shelf

a large shelf will contain two clusters, spaced evenly so distances are uniform between all LED

​

### Power supply

Power is supplied by a USB-A pigtail from the outside.

The pigtail will be hard-wired to the shelf

### LEDs

One Chain for Small Compartment

One Chain for Large Compartment

### Door Switches

## Wiring considerations

Goal: Reduce number of visible wires

Consolidate Switch-and LED wiring

Consider side-feeds to reduce loss induced color imbalances

Use an 8 pin JST connector as only connection to the controller. Feed power into the wardrobe-side of the plug so the only thing that connects to the PCB is that one connector.

Lead USB-C connector out of the wardrobe and have it connected to the PCB for easier updates.

### Wire routing

#### Global Wiring

Each box has its own wiring strand and Data line in the back corner of the box, adjacent to the center wall.

Use one 7-strand wire from CPU compartment to above the closet’s ceiling

At the bottom of the lowest open shelf on the small box, also break out a three strand wire for +5V, GND, LED2 In Data

Route that separate strand through the center divider and route up in the rear corner to the bottom of the ceiling

#### LED Box Wiring

Small Box: Starting at bottom of lowest open shelf

Large Box: Starting at bottom of lowest open shelf

#### Door Switch Wiring

Door switches are wired from above the closet through holes in the ceiling hidden by the switches.

Switch wires are connected to the 7strand wire through the ceiling to the very front of the small box.

One two-wire strand is wired in Manhattan routing to the small box Switch, SW1.

One three-wire strand is wired Manhattan routing to Large Box Switches, SW2 and SW3.

#### Cluster Wiring

Wiring starts in the rear and spans the back row, then circles to the front row to extend across the front row

With multiple clusters on a shelf, they are wired as if they were one big cluster

### Port use on ESP

D5 - Input - SW1, SmallBox

D6 - Output - LED1, SmallBox

D7 - Output - LED2, BigBox

D8 - Input - SW2, BigDoor Left

D9 - Input - SW3, BigDoor Right

VCC - +5V (From power source and to LEDs+Switches)

GND - GND (From Power source and to LEDs)

### Wire color mapping

#### 7 strand riser line

Red: +5V

Black: GND

Green: LED1 Data In/Out

Blue: LED2 Data

Orange: SW1 return

Yellow: SW2 return

White: SW3 return

#### 7 strand riser connector

Red: 1

Black: 4

Green: 2

Blue: 3

Orange: 5

Yellow: 6

White: 7

#### 3 strand LED line

Red: LED1/2 +5V - Red on any riser line

Black: LED1/2 GND - Black on any riser line

Green: LED1/2 In Data / Out Data - Green on any riser line

#### 3 strand LED2 riser

Red: LED2 +5V - Red on 7 Strand riser line

Black: LED2 GND - Black on 7 Strand riser line

Green: LED2 In Data / Out Data - VV on 7 Strand riser line

#### 2 Stranded small box Switch

Red: SW1 +5V - Red on riser line

Black: SW1 return - XX on riser line

#### 3 Stranded large box Switch

Red: SW1 +5V - Red on riser line

Black: SW2 return - YY on riser line

Green: SW3 return - ZZ on riser line

​

#### RJ45 Connectors (male)

View from above, from Left to Right

+5V (On)

GND (On-Wt)

SW1 return (gn-wt)

SW2 return (bl-wt)

SW3 return (bn-wt)

LED1 Data (Gn)

LED2 Data (Bl)

#### JSM connectors

##### 2m (right)

rd = VCC

bk=GND

##### 4m(right)

bk = SW 3

wt = LED2 Data

yl = SW 2

rd = N/A

##### 4f (left)

rd = VCC

bk = GND

wt = LED1 Data

yl = SW1

##### 2f (PWR IN)

rd = VCC

bk = GND