# Requirements collection

## Goal

​

## Compartmentalization

The cabinet is separated into three areas with a glass door each

each door is considered a compartment

each shelf per compartment is separately addressable

?each compartment has its own data line?

power supply is +5V, USB-A or USB-C

each compartment has a power feed to keep brightness stable

## Lighting effects

lights per compartment will try to match the target brightness

if target lower than actual, then fade down to target

if target higher than actual, then fade up to target

## Manual switching

Each door has a switch

opening a door will increase the brightness in that compartment.

closing a door will decrease the brightness of that compartment back to the previous level

manual action will not change target values for brightness

An open door switch will override any automation for that compartment

## Smart integration

Zigbee/ZWave integration with SmartThings Hub will set the brightness target level per compartment.

Alexa integration for easier control through Alexa. Consider prioritization vs. Zigbee based on simplicity

If a door is open, it will set the target for the door-close state

Integrate with WiFi

## Edge cases

in the middle of a fade action a manual or automation event could happen

## Maintainability

aim for a bootstrapper and OTA update-ability

​